Rotherham Metropolitan Borough Council

Community Safety & Street Scene

Highway Inspection & Enforcement Team

Riverside House, Floor 3, Wing A,

Main Street, Rotherham, S60 1AE

Telephone		01709 336003

Email: 		streetworksenforcement@rotherham.gov.uk

SCAFFOLDING REQUIREMENTS

- Prior to placing a scaffold on the highway please contact Streetpride as above.

- A fee is payable per scaffolding / hoarding permit. The charges as from 1st April 2024 are:

- Scaffolding - £220.00 for up to 2 weeks plus £50 per additional 4 week period.

- Hoarding    - £220.00 for the first street plus £50 for each additional street

- Standards should be rendered highly visible (yellow sponging or painted white to first lift). If relevant "wet Paint" Board must be provided.

4.	Minimum height of first lift (if walk through is required) is 2.5 m (8 ft.).

5.	Minimum width of uninterrupted footway to be 1.2 m, if less, then a temporary footway is required to be provided.

6.	Minimum distance between standards and edge of carriageway to be 450 mm (if less, cones etc. are required).

7.	A steady amber lamp is required at each end of the scaffold.

8.	If walk through is provided, then bottom lift must be close boarded.

9.	If required, brickguards and/or double toeboards should be provided.

10.	A plate bearing the name, address and telephone number of the scaffolding erector must be securely fixed to the scaffolding.

11.	The permit holder must inform Rotherham Metropolitan Borough Council when the scaffold is to be removed.

12.	The permit holder must notify all statutory undertakers of his intention to erect the scaffolding.

- Tapping boards and handrails to be provided for the full length of scaffolding except for access point.

- Where necessary, debris netting may be required

Notes

1.	The scaffolding permit can be issued to either the main building contractor or a sub-contractor, e.g. a specialist scaffolder.

2.	Whoever the permit is issued to is responsible for meeting all the conditions of the permit.

3.	If scaffold is found for which no permit has been issued, the person who erected the scaffold is responsible under law.

HOARDING REQUIREMENTS

1.	A white band 150 mm wide and 1.6  m from ground level shall be affixed to the hoarding (around the full length).

2.	The hoarding should be sufficiently lit with steady amber lamps during the hours of darkness.

- A plate bearing the name, address and telephone number of the hoarding erector must be fixed to the hoarding.

4.	The permit holder must notify all statutory undertakers of his intention to erect the hoarding.

A GUIDE TO THE SAFE ERECTION AND DISMANTLING OF SCAFFOLDING

- Introduction

Erecting and dismantling scaffolds remains a high risk activity, not only to those carrying out the work, but to other workers and the general public.  The following guide sets out steps which need to be considered by anyone involved with such work.  It is aimed not only at those directly working in the scaffolding industry but also to clients, planning supervisors and principal contractors.  Set out below are a number of key issues you will need to consider to ensure scaffolding operations are undertaken safely.

- The law and planning for safety

Health and Safety at Work etc. Act 1974.

Construction (Design and Management) Regulations 1994.

Construction (Health, Safety and welfare) regulations 1996.

Management of Health and Safety at Work Regulations 1999.

This guide is not designed to explain the above legislation in detail.  However, it is important to remember that a wide range of people ranging from clients through to the self-employed have legal responsibilities.  In simple terms, the law requires that scaffolding operations are properly planned and then, carrying forward the results of the planning, to ensure the work is carried out safely on site.

- Scaffold Licences

A licence from the local authority is required before a scaffold can be erected on a public highway.  An additional licence is usually needed if you plan to install a protective fan.  A licence may set down standards on such matters as lighting or painting the scaffold or for a fan it may restrict the height at which it is set.  For further advice you should contact the local highway authority.

- Protection of the public

When scaffolding operations are in progress, the public must be excluded from both the area of work and a sufficient area around it.  Steps to ensure this will include:-

- Obtaining a temporary pavement or street closure whilst operations are carried out;

- Undertaking operations in ‘quite’ hours i.e. early morning, at night or at weekends;

- Incorporating fans, crash decks and ‘tunnels’ as early as possible into a scaffold;

- Erecting barriers and signs and diverting the public away from operations;

- Storing scaffold clips and other loose materials safely on the scaffold;

- Not raising or lowering materials over members of the public or other site workers.

Also consider that disabled persons need proper access along pavements covered by scaffold.

- Scaffolders working at height

Scaffolders must follow safe systems of work to prevent people falling.  In particular:-

- When lifting or lowering laterials, scaffolders must be clipped on or working within a handling platform that is fully boarded, with double guard rails and toe boards;

- A minimum 3 boards working platform together with a single guard-rail is provided as erection or dismantling works progress;

- Safety harnesses to be worn at all times by scaffolders and fitted with a 1.75 length lanyard and a 55mm opening hook or similar for one handed operation;

- Harness should be clipped onto a secure anchorage point where falls of 4 metres or more are possible.  A secure anchorage point requires the following minimum conditions:

- The scaffold must be tied in to a sound structure as work progresses;

- Attachment can be made to a ledger, transform or guard-rail supported with load bearing couplers or a transform supported by ledgers in a lift above fixed at both ends by single couplers.

- At least one bay of a scaffold should remain boarded out as work progresses and this should be used for ladder access for the full height of the scaffold;

- Safe ladder access for scaffolders should be incorporated as early as possible into the erection process.

- Scaffolders should not be clambering up and down scaffolds without proper ladder access and safe working platforms provided on each lift being worked on.

- Stability of scaffolds

Each year there are a number of scaffold collapses across the country.  To make sure your scaffold does not collapse you should ensure that:-

- The anchors specified to tie a scaffold to a structure are suitable for the base material and that they are installed correctly;

- Scaffold anchors or ties are installed as erection work progresses.  Conversely, they should not be removed too early during dismantling operations;

- More ties will be needed on a sheeted or netted scaffold to ensure its stability; and

- Scaffolds are not overloaded with equipment, especially tube and fittings, during erection or dismantling operations.

7.	Training, supervision and monitoring

Effective training of scaffolders is possibly the most essential factor in preventing accidents on site.  In addition, do not forget the importance of monitoring the scaffolding contractor.

Clients, principal contractors and others in control should take reasonable steps to ensure that any work being carried out on their site or premises is undertaken safely.  Simple steps to take include:-

- Checking the training levels of scaffolders and who will supervise them on site; and

- Site monitoring of scaffolders to ensure they follow proper safety standards.

8. 	References and further information

Health and Safety in Construction HSG 150 (HSE Books – tel: 01787 881165)

Protecting the public; Your next move HSG 151 (HSE Books – tel: 01787 881165)

National Access and Scaffolding Confederation (NASC) guide SG4:00 ‘The use of fall arrest equipment whilst erecting, altering and dismantling scaffolding’.  (Construction Industry Publications – tel: 0121 722 8200)

BS 5973:1993 Code of practice for access and working scaffolds and special scaffold structures in steel.  (BSI Publication – tel: 020 8996 9001).

BS EN 12811-1:2003. NASC guidance note TG20:13 for tube and fitting.

Reference guide for scaffold / hoarding - 'The New Roads and Street Works Act 1991'  'Safety at Street Works and Road Works'; published by The Stationary Office: Tel 0870 600 5522

- Statutory Undertakers

Attention is drawn to the necessity to notify the statutory undertakers whose names and addresses are attached before the erection of the structure commences in order to enable arrangements to be made to safeguard and maintain access to any apparatus under, or enclosed by, the structure.

Form valid from 1st April 2024 to 31 March 2025