RISK ASSESSMENT FINDINGS

| Department/Service |  |
| --- | --- |
| Date |  |
| Assessor |  |
| Approved By |  |
| Review Date |  |

Relevant Legislation:

The Management of Health and Safety at Work Regulations 1999

RISK ASSESSMENT RECORD

ACTIVITY and/or ENVIRONMENT TO BE ASSESSED: Machinery (Generic)

DATE:

| KEY (People at risk) | Likelihood (L) | Severity (S) | Risk Calculation | Risk Rating |
| --- | --- | --- | --- | --- |
| E = Employee YP = Young Persons P = Public C = Contractors V = Visitors EM = Expectant Mothers | 1. Very Low (rare/very unlikely) 2. Low (unlikely) 3. Medium (could occur/possible) 4. High (likely to occur/probable) 5. Very High (near certain to occur) | 1. Insignificant (nuisance/discomfort) 2. Minor (no lost time) 3. Moderate (time loss) 4. Significant (serious/incapacity to work) 5. Major (Death) | Likelihood x Severity = Rating | 1- 6 LOW RISK Monitor - MEDIUM RISK Monitor, review & reduce risk where possible 14-25 HIGH RISK Further Action Required |

| 1.Hazards Identified and potential harm it could cause | 2. People At Risk | 3.Controls in Place | 4.Risk Rating |  |  |  | 5. Further Action Required/ Recommendations | 6.Target Date for Completion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | S | Score | Risk |  |  |

| Abrasions, cuts, burns, eye injury, entanglement in moving machinery, severance of limbs, crush injury |  | - Appropriate PPE must be worn - Access to the workshop machining areas is strictly restricted to authorised personnel only. - Specific risk assessments for machinery and the generic risk assessment for power hand tools have been completed and safe systems of work issued to operatives. - Guards on the machines must be used. - All workshop equipment is regularly maintained and serviced. - Long hair must be completely covered, and suitable eye protection worn - Weld curtains and anti-flash guards are used when deemed appropriate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electric shocks or burns from using faulty electrical equipment with possible electric seizure or fatality. Abrasions and eye injury. |  | - Appropriate PPE must be worn - Access to the workshops portable electrical equipment is restricted to certified personnel only. - There is a validated PAT testing for electrical safety on each portable appliance. - Electrical cables and plugs should be regularly visually inspected by the user for damage. - Any defective equipment will be reported to the technician-incharge and taken out of use until the repair has been completed. - Electrical equipment is always operated in accordance with manufacturers' instructions. |  |  |  |  |  |  |
| Unfamiliarity with the equipment and/or uncertainty about the job |  | - No one may operate workshop equipment unless they have received a sufficient training and permission from the Teacher/Technician. |  |  |  |  |  |  |
| Manual handling of heavy/bulky objects Back injury, crush injury, Entrapment, damage to goods being lifted or transported |  | - A risk assessment must be completed for lifting heavy and bulky loads that present a risk of injury. - Training in lifting techniques will be provided for workshop staff who undertake the lifting of heavy loads. - Appropriate PPE for lifting operations must be worn. - Appropriate lifting tackle used for specific jobs. - Records of all inspections must be kept in the safety file. |  |  |  |  |  |  |
| Irritating or hazardous vapours, skin sensitisation, dermatitis, burns, asphyxiation |  | - A COSHH risk assessment will be completed for using substances that may present a hazard to health. - A Material Safety Data Sheet (MSDS) must be kept together with a risk assessment in the Workshop Safety File for future reference. - All containers used for storage of hazardous and non-hazardous substances must be suitably labelled indicating their contents - Disposable gloves should be worn if suffering from skin allergies or using irritant material. |  |  |  |  |  |  |
| Slips and trips |  | - Workshop working areas must be kept clear of obstructions and the floor must be kept free from oil and swarf. - Any spillages should be cleaned up immediately. - Any hazards such as trailing cables, defects to floor coverings and faulty lighting etc. should be reported immediately to the technician in-charge. - All areas well lit |  |  |  |  |  |  |
| Fire |  | - Equipment will be switched off when not in use for long periods. - All portable electrical equipment will be tested for electrical safety at correct intervals and labelled with the date of the test. - Flammable substances must be kept away from naked flames and ignition sources. - Flammable substances are kept in an appropriate fire-resistant metal cabinet. - The fire alarm system is installed, maintained and tested. - Workshop users must be acquainted with the Fire Routine Procedure for the building. |  |  |  |  |  |  |
| Excessive Noise |  | - If necessary, a noise risk assessment should be carried out and suitable hearing protection supplied. |  |  |  |  |  |  |
| Gas Cylinder Usage and Storage (Crush injury, Explosion of gas, asphyxiation) |  | - Appropriate storage of gas bottle cylinders by use of rack or cradle system. - Handing and moving cylinders with correct bottle carrier. - Equipment such as regulators, hosing, piping checked at regular intervals and replaced if required. - Required Health & Safety knowledge and safe working practice of equipment being used. |  |  |  |  |  |  |
| Ejection of tool from holder/spindle |  | - Correct PPE must be always worn. - Tooling must be secured before use with correct speeds/feeds selected. Use coolant to prolong tool life, lubricate and prevent overheating of job. - Worn tooling should be discarded or sharpened. - All safety guards must be in place when machine is in operation. |  |  |  |  |  |  |
| Ejection of job from table/Mount |  | - Jobs/fixtures securely fastened with correct clamping systems. - Correct feeds and speeds selected. - All safety guards must be in place when machine is in operation. - No attempt should ever be made to remove swarf whilst the machine is in motion, as it can draw hands and clothing into the machine. - Swarf can also be extremely hot and burns can occur. |  |  |  |  |  |  |
| Machine entanglement while in operation |  | - Loose clothing/jewellery should be secured or not worn - Long hair tied back or netted. - Awareness of emergency stop button location/activation. |  |  |  |  |  |  |
| Injury when loading material on/off machine. |  | - Awareness of manual handling obtained from safety course. - Ask for assistance if needed. - Never overstretch/reach when manoeuvring work pieces. |  |  |  |  |  |  |
| Damaged machine safety equipment such as guards/safety switches |  | - All guards are in place and in good working order - All switches operational and without faults. |  |  |  |  |  |  |
| Airborne Dust Particles |  | - If dust is produced while using material, a dust mask and extractor must be used. |  |  |  |  |  |  |
| Poor lighting and visibility |  | - Work pieces should always have good local lighting from the lathe and area workshop lighting to allow for safe work. |  |  |  |  |  |  |
| Sharp Objects/Material |  | - All PPE provided including gloves, dustcoats and eye protection must be used correctly. |  |  |  |  |  |  |
| Substance released into air |  | - Extractor fan to be used - Open windows - Be aware of any breathing difficulties |  |  |  |  |  |  |
| Vibration |  | - All operatives and supervisors to be provided with hand arm and whole-body vibration training. - Maintenance programme is in place for high use tools that cause hand arm vibration (HAV) - Restrictions in place to limit duration and frequency of using high vibration equipment. |  |  |  |  | High risk equipment to be colour coded and labelled and staff informed. |  |
| Repetitive hand/ arm movement |  | - Regular break taken - Task Variation - Stretch and exercise |  |  |  |  |  |  |

ACKNOWLEDGEMENT OF RISK ASSESSMENT                                                                  Version:

PLEASE READ THE NOTICE BELOW BEFORE SIGNING THIS FORM

I acknowledge that I have read and understood the Risk Assessment/s associated with this task/event/equipment etc

| Forename | Surname | Employee Signature | Date | Manager’s Signature |
| --- | --- | --- | --- | --- |