RISK ASSESSMENT FINDINGS

| Department/Service |  |
| --- | --- |
| Date |  |
| Assessor |  |
| Approved By |  |
| Review Date |  |

Relevant Legislation:

The Management of Health and Safety at Work Regulations 1999

RISK ASSESSMENT RECORD

ACTIVITY and/or ENVIRONMENT TO BE ASSESSED: Use of Kiln

DATE:

| KEY (People at risk) | Likelihood (L) | Severity (S) | Risk Calculation | Risk Rating |
| --- | --- | --- | --- | --- |
| E = Employee YP = Young Persons P = Public C = Contractors V = Visitors EM = Expectant Mothers | 1. Very Low (rare/very unlikely) 2. Low (unlikely) 3. Medium (could occur/possible) 4. High (likely to occur/probable) 5. Very High (near certain to occur) | 1. Insignificant (nuisance/discomfort) 2. Minor (no lost time) 3. Moderate (time loss) 4. Significant (serious/incapacity to work) 5. Major (Death) | Likelihood x Severity = Rating | 1- 6 LOW RISK Monitor - MEDIUM RISK Monitor, review & reduce risk where possible 14-25 HIGH RISK Further Action Required |

| 1.Hazards Identified and potential harm it could cause | 2. People At Risk | 3.Controls in Place | 4.Risk Rating |  |  |  | 5. Further Action Required/ Recommendations | 6.Target Date for Completion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | S | Score | Risk |  |  |

| Heat Metal casing of kiln/firing objects |  | - Hot materials handled with tongs or other suitable tools. - Safety Gloves to be worn when removing items whilst kiln still warm - Kilns situated in working areas should be caged/isolated |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fire |  | - Detailed Fire Risk Assessment conducted and reviewed annually - Rubbish not allowed to accumulate. Located in separate room/area. - Flammable materials are not to be stored in the kiln room. - Clear circulation space is maintained at all times (450mm). - Appropriate fire extinguisher present in vicinity |  |  |  |  |  |  |
| Electrocution Electrical shock Burns |  | - No access to live heating elements - interlocking device where the kiln door cannot be opened before the mains supply is switched off. - Installed by a competent electrician. - Accessible and labelled isolation point. - Extension leads / multiway plugs not to be used to connect kiln. - Visual inspection conducted pre-use. - Kiln must be regularly inspected by a competent person. |  |  |  |  |  |  |
| Use of ceramic materials |  | - Less hazardous chemicals/materials used wherever possible. - No food or drink to be prepared or consumed in any area used for pottery. - Effective hand washing after using ceramics materials and chemicals. - All materials are properly labelled and stored, and appropriate warnings are included in the labelling. - Material Safety Data Sheet for substances obtained from supplier and guidance followed. - COSHH Assessment completed for all hazardous chemicals and control measures implemented. - Immediate cleaning up of any spillage |  |  |  |  |  |  |
| Non authorised use |  | - Used only by competent operators - Students use only under supervision - Kiln sited in a lockable, specialist room which has an external red warning light / warning system. - Written procedures on safe use and emergency procedures posted next to kiln. |  |  |  |  |  |  |
| Ceramic lining/asbestos lining (Ingestion of toxic substances, skin irritation/ sensitization) |  | - Check Site asbestos log information for location of asbestos containing material. - Regular monitoring to ensure that the asbestos is not damaged. - Specialist advice to be sought if work planned on a kiln which could disturb asbestos. |  |  |  |  |  |  |
| Inhalation of dusts including pottery glazes causing harm to health. |  | - Cleaning routines used limit the generation of dusts. - Dry materials stored in sealed containers, spillages cleaned up immediately. - Area regularly cleaned to prevent build-up of clay dust. Floors wet moped daily. - Tables and surfaces to be cleaned whilst clay is damp. - Aprons regularly laundered. |  |  |  |  |  |  |
| Fumes Irritation/sensitisation to respiratory tract |  | - Good natural ventilation e.g. through doors /windows or extraction fan - Do not used lead-based glazes. |  |  |  |  |  |  |
| Glazes to be used with foodstuffs (Ingestion of toxic /harmful substances) |  | - Do not use lead-based glazes. - Ensure glazes or colour do not release metallic compounds when they come into contact with acids contained in food stuffs. - Ensure glazes are used in accordance with the manufacturer’s instructions. |  |  |  |  |  |  |
| Spray application glazes (Irritation/sensitisation to respiratory tract) |  | - Spray glazes should only be used in properly designed spray booth or in outside areas. - Operator should wear a mask. |  |  |  |  |  |  |
| Slips/Trips |  | - Ensure good housekeeping in the kiln room. - Provide wet floor signs when floor is moped. - Dry mop floors after spillages. |  |  |  |  |  |  |
| Manual handling loading / unloading ware |  | - Teachers or technicians should normally load kilns. - Pupils carefully supervised and correct procedures for loading and unloading strictly observed. - Appropriate footwear worn - Mechanical lifting aids available (trolleys etc.) - Heavy items stored at waist level. |  |  |  |  |  |  |

ACKNOWLEDGEMENT OF RISK ASSESSMENT                                                                  Version:

PLEASE READ THE NOTICE BELOW BEFORE SIGNING THIS FORM

I acknowledge that I have read and understood the Risk Assessment/s associated with this task/event/equipment etc

| Forename | Surname | Employee Signature | Date | Manager’s Signature |
| --- | --- | --- | --- | --- |