# Rotherham Metropolitan Borough Council

## Housing Services Asbestos Management Plan Summary

March 2024

Date of next review: March 2025

## CONTENTS

|  |  | Page |
| --- | --- | --- |
| 1 | Introduction | 3 |
| 2 | What is asbestos? | 3 |
| 3 | Managing risk | 4 |
| 4 | The regulations | 4 |
| 5 | How is asbestos assessed? | 5 |
| 6 | Who has responsibility for managing asbestos? | 6 |
| 7 | Managing asbestos in Council buildings | 6 |
| 8 | The Council's asbestos register | 8 |
| 9 | Informing residents and leaseholders about asbestos. | 9 |
| 10 | Informing employees about asbestos | 9 |
| 11 | Performance Monitoring | 9 |

### 1 INTRODUCTION

This is a summary of the Council's Housing Asbestos Management Plan designed for
non-specialists and the general public. The full Plan is available upon request.

The aim of the Asbestos Management Plan is to effectively manage the risks
presented by asbestos containing materials (ACM) in the Council's Housing Revenue
Account (HRA) stock portfolio. These properties are mostly rented to residents as
Council houses or used as neighbourhood centres.

The Council, as a responsible employer and property owner, is aware of its legal
duties to employees, contractors, tenants, their households', and visitors', to provide
information about asbestos, and its duty to manage asbestos within its properties.

When dealing with asbestos, the Council aims to create a safe working environment.
To achieve this, a structured risk-based approach is taken to managing and
controlling materials containing asbestos. This includes the use of licensed and
accredited specialists, planned surveys, re-inspections, and maintenance of records.

The Council recognises the hazards relating to asbestos and the risks to health that it
poses. To fulfil its moral, social, and legal obligations a detailed plan has been
developed. This document is a summary of that extensive plan.

### 2 WHAT IS ASBESTOS

Asbestos is a common name for a group of naturally occurring fibrous materials
which were commonly used in a range of equipment and buildings. Asbestos was
prohibited in the UK in 1999 but was used in the building industry, particularly
between 1950 and 1980.

Legislation relating to asbestos management identifies six types of asbestos:

· Chrysotile (white)
· Amosite/Grunerite (brown)
· Crocidolite (blue)
· Fibrous Tremolite
· Fibrous Actinolite
· Fibrous Anthophyllite

Of these six types of asbestos, three are typically found in residential properties -
Chrysoltile (white asbestos), Amosite/Grunerite (brown asbestos) and Crocidolite
(blue asbestos).

### 3 MANAGING RISK

Asbestos is only dangerous when disturbed. If it is safely managed and contained, it
does not present a health hazard.

Risk is greatest when:
· Work is taking place on unfamiliar sites.
· People are not trained on asbestos awareness.
· Asbestos surveys are not carried out, or not shared.

Basic principles to consider when managing asbestos:
· Do not start intrusive work to a building without checking the Asbestos Register
· Do not remove asbestos unnecessarily - removing it can be more dangerous
than leaving it in place and managing it

The duty to manage is all about putting in place practical steps to protect people
from the risk of exposure to asbestos fibres - it is not about removing all asbestos.

### 4 THE REGULATIONS

The Control of Asbestos Regulations 2012 (CAR) is the main legislation relating to
asbestos management. Although the legislation excludes residential properties
from Regulation 4 and the Duty to manage, the Council has chosen to apply the
regulations to all residential and non-domestic assets.

The Control of Asbestos Regulations 2012 has various Approved Codes of Practice
(ACoPs) and guidance notes including, but not limited to:
· L143 Managing and Working with Asbestos
· HSG264 The Survey Guide
· HSG210 Asbestos Essentials
· HSG247 The Licensed Contractors Guide

The duty to manage asbestos is contained in regulation 4 of CAR 2012. It requires
the duty holder to:
a) take reasonable steps to find out if there are materials containing asbestos...
h) provide information on the location and condition of the materials to anyone
liable to work on or disturb them